Color Zoom Camera

USER’S MANUAL

Version 2.0.0

Thank you for purchasing our product. If there is any question or request, please do not hesitate to contact dealer.

This manual is applicable to DS-2CZ232P/N, DS-2CM232P/N, DS-2CZ252P/N, DS-2CM252P/N, DS-2CZ282P, and DS-CZ292P series color zoom cameras.

This manual may contain several technically incorrect places or printing errors, and the content is subject to change without notice. The updates will be added into the new version of this manual. We will readily improve or update the products or procedures described in the manual.

Safety Instruction

These instructions are intended to ensure that user can use the product correctly to avoid danger or property loss.

The precaution measure is divided into “Warnings” and “Cautions”

Warnings: Serious injury or death may cause if any of the warnings is neglected.

Cautions: Injury or equipment damage may cause if any of the cautions is neglected.

| Warnings Follow these safeguards to prevent serious injury or death. | Cautions Follow these precautions to prevent potential injury or material damage. |
| --- | --- |

Warnings

1.  In the use of the product, you must be strict compliance with the electrical safety regulations of the nation and region.

2.  Input voltage should meet both the SELV(Safety Extra Low Voltage) and the Limited Power Source with AC 24V or DC 12V according to the IEC60950－1 standard. Please refer to technical specifications for detail information.

3.  Do not connect several devices to one power adapter as adapter overload may cause over-heat or fire hazard.

4.  Please make sure that the plug is firmly connected on the power socket.

5.  When the product is mounted on wall or ceiling, the device shall be firmly fixed.

6.  If smoke, odor or noise rise from the device, turn off the power at once and unplug the power cable, and then please contact the service center.

7.  If the product does not work properly, please contact your dealer or the nearest service center. Never attempt to disassemble the camera yourself. (We shall not assume any responsibility for problems caused by unauthorized repair or maintenance.)

Cautions：

- Make sure the power supply voltage is correct before using the camera.

- Do not drop the camera or subject it to physical shock.

- Do not touch CCD (Charge Coupled Device) modules with fingers. If cleaning is necessary, use clean cloth with a bit of ethanol and wipe it gently. If the camera will not be used for an extended period, please turn on the lens cap to protect the CCD from dirt.

- Do not aim the camera at the sun or extra bright places. A blooming or smear may occur otherwise (which is not a malfunction however), and affecting the endurance of CCD at the same time.

- The CCD may be burned out by a laser beam, so when any laser equipment is on using, make sure that the surface of CCD will not be exposed to the laser beam.

- Do not place the camera in extremely hot, cold(the operating temperature shall be－10℃～＋60℃ ), dusty or damp locations, and do not expose it to high electromagnetism radiation.

- To avoid heat accumulation, good ventilation is required for operating environment.

- Keep the camera away from liquid while on using.

- While on a delivery, the camera shall be packed in its original packing, or packing of the same texture.

- Regular part replacement: a few parts (e.g. electrolytic capacitor) of the equipment shall be replaced regularly according to their average enduring time. The average time varies because of differences between operating environment and using history, so regular checking is recommended for all the users. Please contact with your dealer for more details.

Table of Contents

C H A P T E R 1	1

Introduction	1

1.1 Product Features	2

1.2 Side Plate of the Camera	2

1.3 Rear Plate of the Camera	3

1.4 Chief Introduction about Function	3

C H A P T E R 2	5

Installation	5

2.1 Lens	6

2.2 Rear Plate Definition	6

2.3 Power Supply	6

C H A P T E R 3	7

Menu Description of 232 series	7

3.1 Keys Introduction	8

3.1.1 Setting buttons	8

3.1.2 Zoom/Focus Wire Remote Control	8

3.1.3 Alarm in, GND, Alarm output, Alarm output Com	9

3.1.4 Power supply	9

3.1.5 Interface Definition	9

3.2 OSD Menu Configuration	10

3.2.1 Menu Structure	11

3.2.2 Exposure	12

3.2.3 Auto Gain Control	12

3.2.4 White Balance	12

3.2.5 Auto Focus	13

3.2.6 Day / Night	13

3.2.7 Motion Detection	14

3.2.8 Special Functions	14

3.2.9 Exit	16

C H A P T E R 4	17

Menu Description of 252 series	17

4.1 Keys Introduction	18

4.1.1 Setting buttons	18

4.1.2 Zoom/Focus Wire Remote Control	18

4.1.3 Alarm in, GND, Alarm output, Alarm output Com	19

4.1.4 Power supply	19

4.1.5 Interface Definition	19

4.2 OSD Menu Configuration	20

4.2.1 Menu Structure	20

4.2.2 Shutter	22

4.2.3 Auto Gain Control	22

4.2.4 White Balance	23

4.2.5 Auto Focus	23

4.2.6 Day / Night	24

4.2.7 Motion Detection	25

4.2.8 Special Fuctions	25

4.2.9 Exit	27

Appendix Specification	28

Table 1 DS-2CZ232P/N	28

Table 2 DS-2CM232P/N	29

Table 3 DS-2CZ252P/N	30

Table 4 DS-2CM252P/N	31

Table 5 DS-2CZ282P	32

Table 6 DS-2CZ292P	33

# C H A P T E R 1

Introduction

1.1 Product Features

DS-2CZ232P(DS-2CM232P),DS-2CZ252P（DS-2CM252P）series are high performance cameras which adopt 1/4" SONY Interline Transfer Super HAD CCD and advanced print circuit board design technology, and possess of high resolution, lower distortion, and lower noise features. They are extremely suitable for surveillance system and image process system.

Features:

• Digital signal process camera (DSP), 480TVL;

• DS-2CZ232P and DS-2CM232P series hold 22X optics variable focus and auto focus functions;

• DS-2CZ252P and DS-2CM252P series hold 23X Optics variable focus and auto focus functions;

• DS-2CZ232P and DS-2CM232P series with Iris F1.6, f=3.9-85.8mm;

• DS-2CZ252P and DS-2CM252P series with Iris F1.6, f=3.84-88.4mm;

• Support automatic electron shutter, auto iris/ manual iris, automatic/manual white balance;

• High S/N ratio, support color black/white auto switch function;

• High color rendition;

• Support back light compensation, automatic electron gain;

• Support motion detection;

•Support RS-485/wire remote control;

• Support OSD menu and parameters configuration;

• Support internal synchronization;

• Support PAL standard;

• DC12V±10% power supply;

## 1.2 Side Plate of the Camera

Figure 1.1 DS-2CZ232P side plate

Figure 1.2 DS-252P side plate

## 1.3 Rear Plate of the Camera

Figure 1.3 Rear plate of DS-232P and DS-252P

## 1.4 Chief Introduction about Function

Motion Detection: In the surveillance area, the appearing motion object will trigger alarm. User can select suitable sensitivity level with corresponding environment.

Color B/W auto switch: The camera will display color image and become monochrome automatically at night.

AGC: When object being shot look dim, please turn on AGC. It is propitious to enhance brightness. To output standard video under different illumination, the range of auto gain control must be large enough. When under low illumination, AGC will increase camera’s sensitivity, and output bright and clear video.

S/N ratio: It is the ratio of Signal voltage and noise voltage. The ratio is larger, the effect of noise is less, and the image is more spotless.

OSD (On Screen Display): The camera supports friendly menu interface on the screen, and it’s convenient for user selecting expected function.

White Balance: Because the camera could not adjust the color temperature according to the light alteration as the eyes, that will cause color deviation. White balance is the white rendition ability of the camera to adjust the problem according to the environment automatically.

ICR Auto Switch: The filter will filter infrared light during the daytime and change to normal filter at night to ensure a high sensitivity and clear image.

BLC:  If there is glaring light in the background of the object being shot, the object will look very dim. In this situation, turning on BLC function will make the object look bright and clear, but the background scene will be overexposure.

BLC OFF                                BLC ON

# C H A P T E R 2

# Installation

Before mounting, please make sure that the device in the package is in good condition and all the assembly parts are complete.

## 2.1 Lens

The lens of the automatic focus camera were installed inside of the camera, 232 series cameras with f=3.9-85.8mm, 252 series cameras with f=3.84-88.4mm.

## 2.2 Rear Plate Definition

Fig. 2.1 Rear plate definition of DS-2CZ232P and DS-2C252P

## 2.3 Power Supply

Before electrify the camera, please check the power supply. The power supply of the camera should be DC12V±10%. Please refer to the specification for the details.

# C H A P T E R 3

# Menu Description of 232 series

The menu functions of the DS-2CZ232P/N will be introduced in this chapter. The first part is the instruction of keys and menu structure, the second part is to introduce the menu functions of all the items.

## 3.1 Keys Introduction

In this section, it will introduce the keys on the rear plate.

3.1.1 Setting buttons

Fig 3.1 The keys of camera

There are two work modes for the setting buttons: Common and Menu modes. Each of them has its own function.

Common mode:

T/W button: Tele/ wide zoom operation.

N/F button: Manual variable zoom, near/ far.

Menu button: Menu mode switch. Pressing MENU key for 2 seconds, it will enter menu setting mode.

Menu mode:

T/W button: Move the cursor up/down.

N/F button: Move the cursor left/right.

Menu button: Select options, and confirm the current value.

### 3.1.2 Zoom/Focus Wire Remote Control

“”  RS-485, support PELCO-D protocol.

Default setting requirements:

- Signal transmission speed:2400bit/s

- Protocol: PELCO-D

- Data bit: 8bit

- Stop bit: 1bit

- Parity bit: None

### 3.1.3 Alarm in, GND, Alarm output, Alarm output Com

“”  When motion detection module detects moving objects, it will trigger alarm signal.

### 3.1.4 Power supply

“”  Before connecting power supply adapter to camera, please check the anode and cathode.

### 3.1.5 Interface Definition

DS-2CZ232P interface definition:

| Z “” | Zoom (Wire Control Port) |
| --- | --- |
| C “” | COM (Wire Control Port) |
| F “” | Focus (Wire Control Port) |
| IN/GND“” | Alarm Input/GND |
| OUT/COM “” | Alarm Output/COM |
| D+/D- “” | 485+/- |
| GND “” | Common GND |

Fig 3.2 DS-2CZ232P interface definition table

Note: Z/F working voltage is: +3V to +12V, - 3V to -12V.

Alarm output: Open Collector: Drive load is DC24V 45mA Max.

DS-2CM232P interface definition:

| Pin number | Name | Voltage |
| --- | --- | --- |
| 1 | Rx | CMOS 5V,series receive pin |
| 2 | Tx | CMOS 5V,series send pin |
| 3 | GND | Series GND |
| 4 | DC IN | 12VDC±10% |
| 5 | GND | Power supply GND |
| 6 | CVBS OUT | 1.0V±0.2V Video Output |
| 7 | GND | Composite Video GND |
| 8 | NC |  |
| 9 | GND |  |

Fig 3.3 DS-2CM232P interface definition table

## 3.2 OSD Menu Configuration

In this section, it will introduce the menu of DS-2CZ232P. Pressing   button for two seconds will enter main menu. The left is Menu items; the right is Menu items value. “…” means this option has a submenu, “---” means this option cannot be set.

Main Menu

CAM ID               001

Shutter               Auto

AGC                 ---

WB                  Auto1

AF                   Auto…

D/N                  Auto…

Motion D             Close

Special               …

Exit                  Abort

Fig 3.4 Main Menu

### 3.2.1 Menu Structure

| 1. CAM ID | OFF/001~254，Edit camera ID number |
| --- | --- |
| 2. Exposure | Auto：Auto mode, Auto Iris,AGC , AES 1/50 AI…：Auto Iris, AGC, AES (1/50~1/10000) AE…：AES,AGC(Low、High、Close),AI(F1.6～F5.6) Manual…：Shutter, IRIS,AGC can be set by manual. |
| 3. AGC | Low/High/Close, if the exposure mode is Auto or AI, AGC will not be set ,it display “---” |
| 4. WB | ATW1/ATW2/Auto/Manual… |
| 5. AF | Auto…:To enter submenu and set the focus mode, sensitivity and the least focus distance --- (1cm、50cm、1m) Manual One focus: triggered by button once, then turn to manual mode. |
| 6. Color/ B/W | Auto…:Color and B/W auto switch mode Color… B/W… |
| 7. Motion D | Close/Open…:To enter submenu and set motion detection sensitivity and areas |
| 8. Special | Press Menu button enter special function submenu（CN/EN,BLC,RS-485,POS/NEG, ZSpeed, Definition, Line Control ） |
| 9. Exit | Abort/Save/Default |

Fig 3.5 Main Menu set

| (Special) |  |
| --- | --- |
| 1. CN/EN | EN |
| 2. BLC | Close: Turn off BLC Default…: Default BLC mode User…:user self-definition, the BLC areas position and size can be set |
| 3. RS-485 | 485 communication parameter,to enter submenu to set RS-485 communication protocols, Baud and RS-485 address |
| 4. POS/NEG | ＋/- |
| 5. ZSpeed | Zoom speed:1～4 class |
| 6. Define | Definition ,1～5 class |
| 7. Back |  |

Fig 3.6 Special Menu set

Main Menu

CAM ID                        OFF/001—254

Shutter                         Auto/AI…/AE…/Manual…

AGC                           Low/High/Close

WB                            ATW1/ATW2/Auto/Manual…

AF                             Auto…/Manual/One Focus

Color/B/W                      Color…/Auto…/B/W…

Motion D                       Open…/Close

Special                         …

Exit                            Default/Abort/Save

Fig 3.7 Main Menu structure

### 3.2.2 Exposure

The iris can control output video signal according to the different intensity of light. (This product is already equipped with iris.) The “F”(F1.6～F5.6) value of the iris can be set by user.

1. AI…

Manual shutter  1/50、1/120、1/250、1/500、1/1000、1/2000、1/4000、1/10000

2. AES…

Manual Iris   F1.6-----------1 /F5.6-----------1

3. Manual...

Manual shutter  1/50、1/120、1/250、1/500、1/1000、1/2000、1/4000、1/10000

Manual Iris   F1.6-----------1/F5.6----------1

4. Auto

Fig 3.8 Exposure setup

Under AES and Manual mode, Iris can be set from F1.6 to F5.6 in exposure menu.

Under AI and Manual mode, shutter can be set from 1/50s to 1/10Ks (8 options).

### 3.2.3 Auto Gain Control

When enable AGC in the situation of Manual and AES, there are three AGC options: Low, High and Off.

When under AI and Manual modes, AGC is not available.

### 3.2.4 White Balance

In the “WB” options, user can set the white balance function to ensure it can render normal white in any color temperature. If selecting the ATW mode, the camera will continue monitoring the color temperature according to the surrounding light and adjust WB based on the color temperature. Including ATW1, ATW2, Auto, and Manual modes.

WB             ATW1/ATW2/Auto/Manual…

Setup           User/3200K/6300K

RED           （+1F）--------|----------

Blue            (-53 )  -|------------------

Back

Fig 3.9 white balance setup

ATW1: Camera recover color automatically based on TTL algorithm, temperature range is approximate from 2500K to 9300K.

ATW2: Camera recover color automatically based on TTL algorithm, temperature range is approximate from 2000K to 10000K.

ATC: Adjust under steady color temperature environment. After selecting ATC, the white balance will take effect to the current scene, and adjust the temperature to 6500K.

Manual: Adjust red and blue values to setup white balance.

### 3.2.5 Auto Focus

In the options of AF, user can select the Focus modes including Auto, Manual, One focus.

Auto: it can accomplish continuous surveillance without manual adjustment.

Manual: User can manual focus.

One focus: Press the button to focus once, then it will be the same as manual mode.

AF Setup

Mode                      Normal/Interval

Sense                    High/Mid/Low

Near Lim                  50cm/1m/1cm

Back

Figure 3.10 AF Setup

### 3.2.6 Day / Night

Optional D/N modes: Auto D/N, Day, Night mode.

Note: The Auto mode requires meeting any of the following conditions:

(a). The exposure mode should be Auto or AI.

(b). If the exposure mode is manual or AE, the AGC should be set as High.

### 3.2.7 Motion Detection

In Motion Detection, the Motion D functions, sensitivity and motion areas can be set by manual. After the setup, user can detect the motion situation of the intruder.

The motion D has two work modes: When Motion D is Close, if alarm input occurs, the alarm output will directly respond. When Motion D is Open, the camera will auto detect monitoring area, and output alarm signal.

### 3.2.8 Special Functions

User can set the CN/EN language, RS-485, BLC, Zoom Speed, Definition, POS/NEG and Line control.

Special Function

CN/EN                        EN

BLC                          User…/Close/Default…

RS-485                       …

POS/NEG                     +/-

ZSpeed                       1/ 2/ 3/ 4

Define                        1/ 2/ 3/ 4/ 5

Line                          Mode1/Mode2

Back

Fig 3.11 Special Function sets

CN/EN Setup:

Move cursor to “CN / EN” through up / down, and press left / right to select CN or EN.

BLC Setup:

The options have User…/Close/Default…

Note: This function is only available when “Auto” selected in Exposure. The setup will retain when other options selected in Exposure.

When “Default” is selected, the camera will adjust according to the default region. Press “MENU” to see the default back light area, and press “MENU” again to exit.

When select “USER”, user can customize the back light area.

When entering adjust status, use up/down/left/right to adjust the back light area position. Press “MENU” once to release, use up/down/left/right to change to area adjustment status, press “MENU” for 2 min to exit.

Right,Up

Button

Figure 3.12 BLC

When select “Off”, camera will not take back light compensation.

RS-485

Press MENU button to enter RS485 setup interface:

RS485

Protocol   PELCO-D

Baud      2400     (1200, 2400, 4800, 9600, 19200, 38400, 115200 optional)

Address    001      (000 to 254 optional)

Back

Figure 3.13 RS-485 Setup Interface

POS/ NEG

Move cursor to “POS / NEG”, and press left / right to select POS or NEG. Selecting “+”, the video will be normal, while selecting “-”, the video will be negative.

Zoom Speed

Move cursor to “Z Speed”, and press left/right to select from “1/2/3/4”. 1 means the slowest speed, and 4 means the fastest speed.

Definition

Move cursor to “Definition”, and press left/right to select High, Middle, 低中, and low to adjust the image definition.

Line Control

Move cursor to “Wire Remote Control”, and press left/right to select “Mode1/Mode2”.

Mode1 is: When Vz-Vc=12V, wide-angle direction zooming. When Vz-Vc=-12V, tele direction zooming;

Mode 2 is on the opposite.

### 3.2.9 Exit

Exit option is used for quit setting Menu, and return to normal work mode.

Abort: Neglect all modifications, and resume the original setup.

Save: Save all modifications.

Default: Neglect all modifications, resume the default factory setup.

# C H A P T E R 4

# Menu Description of 252 series

The menu functions of the DS-2CZ232P will be introduced in this chapter. The first part is the instruction of keys and menu structure, the second part is to introduce the menu functions of all the items.

## 4.1 Keys Introduction

In this section, it will introduce the keys on the rear plate.

### 4.1.1 Setting buttons

Fig 4.1 The keys of camera

There are two work modes for the setting buttons: Common and Menu modes. Each of them has its own function.

Common mode:

T/W button: Tele/ wide zoom operation.

N/F button: Manual variable zoom, near/ far.

Menu button: Menu mode switch. Pressing MENU key for 2 seconds, it will enter menu setting mode.

Menu mode:

T/W button: Move the cursor up/down.

N/F button: Move the cursor left/right.

Menu button: Select options, and confirm the current value.

### 4.1.2 Zoom/Focus Wire Remote Control

“” RS-485, support PELCO-D protocol.

Default setting requirements:

- Signal transmission speed:2400bit/s

- Protocol: PELCO-D

- Data bit: 8bit

- Stop bit: 1bit

- Parity bit: None

### 4.1.3 Alarm in, GND, Alarm output, Alarm output Com

“” When motion detection module detects moving objects, it will trigger alarm signal.

### 4.1.4 Power supply

“” Before connecting power supply adapter to camera, please check the anode and cathode.

### 4.1.5 Interface Definition

DS-2CZ252P interface definition:

| Z “” | Zoom (Wire Control Port) |
| --- | --- |
| C “” | COM (Wire Control Port) |
| F “” | Focus (Line Control Port) |
| IN/GND“” | Alarm Input/GND |
| OUT/COM “” | Alarm Output/COM |
| D+/D- “” | 485+/- |
| GND “” | Common GND |

Fig 4.2 DS-2CZ252P interface definition table

Note: Z/F working voltage is: +3V to +12V, -3V to -12V.

Alarm output: Open Collector: Drive load is DC24V 45mA Max.

DS-2CM252P interface definition

| Pin number | Name | Voltage |
| --- | --- | --- |
| 1 | Rx | CMOS 5V,series receive pin |
| 2 | Tx | CMOS 5V,series send pin |
| 3 | GND | Series GND |
| 4 | DC IN | 12VDC±10% |
| 5 | GND | Power supply GND |
| 6 | CVBS OUT | 1.0V±0.2V Video Output |
| 7 | GND | Composite Video GND |
| 8 | NC |  |
| 9 | GND |  |

Fig 4.3 DS-2CM252P interface definition table

## 4.2 OSD Menu Configuration

In this section, it will introduce the menu of DS-2CZ252P. Pressing   button for two seconds will enter main menu. The left is Menu items; the right is Menu items value. “…” means this option has a submenu, “---” means this option cannot be set.

Main Menu

CAM ID               001

Shutter               Auto

AGC                 ---

WB                  Auto1

AF                   Auto…

D/N                  Auto…

Motion D             Close

Special               …

Exit                  Abort

Figure 4.4 Main Menu

### 4.2.1 Menu Structure

| 1. CAM ID | OFF/001~254，Edit camera ID number |
| --- | --- |
| 2. Shutter | Auto：Auto mode, Auto Iris,AGC , AES 1/50 AI…：Auto Iris, AGC, AES (1/50~1/10000) AE…：AES,AGC(Low、High、Close),AI(F1.6～F5.6) Manual…：Shutter, IRIS, AGC can be set by manual. |
| 3. AGC | Low/High/Close, if the exposure mode is Auto or AI, AGC will not be set ,it display “---” |
| 4. WB | ATW1/ATW2/Auto/Manual… |
| 5. AF | Auto…:To enter submenu and set the focus mode, sensitivity and the least focus distance --- (1cm、50cm、1m) Manual One focus: triggered by button once, then turn to manual mode. |
| 6. D/N | Auto…: Day and night auto switch mode Day…: Enter sub menu to set saturation Night…: Enter sub menu to set color synchronization |
| 7. Motion D | Close/Open…:To enter submenu and set motion detection sensitivity and areas |
| 8. Special | Press Menu button enter special function submenu（CN/EN,BLC,RS-485,POS/NEG, ZSpeed, Definition, Line ） |
| 9. Exit | Abort/Save/Default |

Fig 4.5 Main menu set

| (Special) |  |
| --- | --- |
| 1. CN/EN | EN |
| 2. BLC | Close: Turn off BLC Default…: Default BLC mode User…:user self-definition, the BLC areas position and size can be set |
| 3. RS-485 | 485 communication parameter to enter submenu to set RS-485 communication protocols, Baud and RS-485 address |
| 4. POS/NEG | ＋/- |
| 5. ZSpeed | Zoom speed:1～4 class |
| 6. Define | Definition ,1～5 class |
| 7. Line | Mode1/Mode2 |
| 8. Back |  |

Fig 4.6 Special Menu set

Main Menu

CAM ID                        OFF/001—254

Shutter                         Auto/AI…/AE…/Manual…

AGC                           Low/High/Close

WB                            ATW1/ATW2/Auto/Manual…

AF                             Auto…/Manual/One Focus

D/N                            Day…/Auto…/Night…

Motion D                       Open…/Close

Special                         …

Exit                            Default/Abort/Save

Fig 4.7 Main Menu structure

### 4.2.2 Shutter

The iris can control output video signal according to the different intensity of light. (This product is already equipped with iris.) The “F”(F1.6～F5.6) value of the iris can be set by user.

1、AI…

Manual shutter  1/50、1/120、1/250、1/500、1/1000、1/2000、1/4000、1/10000

2、AES…

Manual Iris   F1.6-----------1 /F5.6-----------1

3、Manual...

Manual shutter  1/50、1/120、1/250、1/500、1/1000、1/2000、1/4000、1/10000

Manual Iris   F1.6-----------1/F5.6----------1

4、Auto

Fig 4.8 Exposure setup

Under AES and Manual mode, Iris can be set from F1.6 to F5.6 in exposure menu.

Under AI and Manual mode, shutter can be set from 1/50s to 1/10Ks (8 options).

### 4.2.3 Auto Gain Control

When enable AGC in the situation of Manual and AES, there are three AGC options: Low, High and Off.

When under AI and Manual modes, AGC is not available.

### 4.2.4 White Balance

In the white balance options, user can set the white balance function to ensure it can render normal white in any color temperature. If selecting the ATW mode, the camera will continue monitoring the color temperature according to the surrounding light and adjust WB based on the color temperature. Including ATW1, ATW2, Auto, and Manual modes.

WB             ATW1/ATW2/Auto/Manual…

Fig 4.9 white balance setup

ATW1: Camera recover color automatically based on TTL algorithm, temperature range is approximate from 2500K to 9300K.

ATW2: Camera recover color automatically based on TTL algorithm, temperature range is approximate from 2000K to 10000K.

ATC: Adjust under steady color temperature environment. After selecting ATC, the white balance will take effect to the current scene, and adjust the temperature to 6500K.

Manual: Adjust red and blue values to setup white balance.

Setup           User/3200K/6300K

RED           （+1F）--------|----------

Blue            (-53 )  -|------------------

Back

Figure 4.10 MWB setup

### 4.2.5 Auto Focus

In the options of AF, user can select the Focus modes including Auto, Manual, One focus.

Auto: it can accomplish continuous surveillance without manual adjustment.

Manual: User can manual focus.

One focus: Press the button to focus once, then it will be the same as manual mode.

AF

Mode                      Normal/Interval

Sense                    High/Mid/Low

Near Lim                  50cm/1m/1cm

Back

Figure 4.11 Auto Focus Setup

### 4.2.6 Day / Night

Optional day to night modes: Auto day switch to night, day mode, night mode.

Auto day switch to night: Camera will switch between color and B/W according to the environment illumination. Under low illumination condition, IR filter will move away and switch to B/W mode to improve the sensitivity.

Day Mode: IR filter will move into, the image back to normal status.

Night Mode: IR filter will move away, the image will change to black and white status to improve sensitivity.

Night Setup

Burst             Off/On

Back

Figure 4.12 Night setup

Burst:

Off: Do not output chroma signal and black and white composite video signal

On: Output chroma signal.

Day Setup

Chroma         High/Mid/Low

Back

Fig 4.13 Day setup

Chroma: Adjust chroma parameters, has low/mid/high options.

Auto D/N

Value           (2)------l-----

C/BW      2s/3s/5s/10s/15s/20s/25s/30s

BW/C      2s/3s/5s/10s/15s/20s/25s/30s

Back

Figure 4.14 Auto D/N setup

Value: The threshold for day mode change to night mode.

C/BW: The delay time for color change to black and white mode.

BW/C: The delay time for black and white mode change to color mode.

### 4.2.7 Motion Detection

In Motion Detection, the Motion D functions, sensitivity and motion areas can be set by manual. After the setup, user can detect the motion situation of the intruder.

The motion D has two work modes: When Motion D is Close, if alarm input occurs, the alarm output will directly respond. When Motion D is Open, the camera will auto detect monitoring area, and output alarm signal.

### 4.2.8 Special Fuctions

User can set the CN/EN language, RS-485, BLC, ZoomSpeed, Definition ,POS/NEG, Line by manual.

Special Function

CN/EN                        EN

BLC                          User…/Close/Default…

RS-485                       …

POS/NEG                     +/-

ZSpeed                       1/ 2/ 3/ 4

Define                        1/ 2/ 3/ 4/ 5

Line                          Mode1

Back

Figure 4.15 Special Function sets

CN/EN Setup:

Move cursor to “CN / EN” through up / down, and press left / right to select CN or EN.

BLC Setup:

The options have User…/Close/Default…

Note: This function is only available when “Auto” selected in Exposure. The setup will retain when other options selected in Exposure.

When “Default” is selected, the camera will adjust according to the default region. Press “MENU” to see the default back light area, and press “MENU” again to exit.

When select “USER”, user can customize the back light area.

When entering adjust status, use up/down/left/right to adjust the back light area position. Press “MENU” once to release, use up/down/left/right to change to area adjustment status, press “MENU” for 2 min to exit.

Right Up

Button

Figure 4.16 BLC

When select “Off”, camera will not take back light compensation.

RS-485

Press MENU button to enter RS485 setup interface:

RS485

Protocol   PELCO-D

Baud     2400     （1200、2400、4800、9600、19200、38400、115200 optional）

Address       001      （000---254 optional）

Back

Figure 4.17 RS-485 Setup Interface

POS/ NEG

Move cursor to “POS / NEG”, and press left / right to select POS or NEG. Selecting “+”, the video will be normal, while selecting “-”, the video will be negative.

Zoom Speed

Move cursor to “Z Speed”, and press left/right to select from “1/2/3/4”. 1 means the slowest speed, and 4 means the fastest speed.

Definition

Move cursor to “Definition”, and press left/right to select from 5 levels to adjust the image definition.

Line Control

Move cursor to “Wire Remote Control”, and press left/right to select “Mode1/Mode2”.

Mode1 is: When Vz-Vc=12V, wide-angle direction zooming. When Vz-Vc=-12V, tele direction zooming;

Mode 2 is on the opposite.

### 4.2.9 Exit

Exit option is used for quit setting Menu, and return to normal work mode.

Abort: Neglect all modifications, and resume the original setup.

Save: Save all modifications.

# Appendix Specification

## Table 1 DS-2CZ232P/N

| Parameter Model |  | DS-2CZ232P/N |
| --- | --- | --- |
| Image Sensor |  | 1/4 inch SONY Interline Transfer Super HAD CCD |
| Signal System |  | PAL/NTSC |
| Effective Pixels |  | PAL: 752 (H) × 582 (V) NTSC: 768 (H) × 494 (V) |
| Synchronization |  | Internal Synchronization |
| Horizontal Resolution |  | 480 TVL |
| Video Output |  | 1Vp-p Composite Output (75Ω/BNC) |
| S/N Ratio |  | More than 50dB |
| Day & Night (Electronic) |  | Auto / Color / B&W |
| Min. Illumination |  | 0.1Lux @ F1.6 |
| Lens |  | 22X Optical Zoom |
|  |  | F1.6, f=3.9 ~ 85.8mm |
|  |  | Min. Working Distance: 10-1000mm(Wide-Tele) |
| Menu | Camera ID | On / Off (001-254, fixed position) |
|  | Language | English, Chinese |
|  | Iris | Auto / Manual |
|  | Electronic Shutter | 1/50(1/60)s to 1/100,000s |
|  | Auto Gain | Low / High / Off |
|  | Auto Focus | Auto / Manual / Once |
|  | Auto White Balance | ATW1 / ATW2 /Auto / Manual( Pre-set 3200K, Pre-set 5600K, R / G Gain Correction) |
|  | Motion Detection | On / Off (Zone, Sensitivity) |
|  | Special Function | BLC, POS/NEG, Zoom Speed (4 levels), Definition |
|  | Communication Interface | RS-485, PELCO-D, Baud Rate, Address, PTZ Control |
| Power Supply |  | 12VDC, ±10% |
| Working Temperature |  | -10℃ ~ 60℃ |
| Power Consumption |  | 4.5W |
| Dimension (mm) |  | 64 × 61 × 112.5(2.53” × 2.42” × 4.46”) |
| Weight |  | 375g (0.82lbs) |

## Table 2 DS-2CM232P/N

| Parameter Model |  | DS-2CM232P/N |
| --- | --- | --- |
| Image Sensor |  | 1/4 inch SONY Interline Transfer Super HAD CCD |
| Signal System |  | PAL/NTSC |
| Effective Pixels |  | PAL: 752 (H) × 582 (V) NTSC: 768 (H) × 494 (V) |
| Synchronization |  | Internal Synchronization |
| Horizontal Resolution |  | 480 TVL |
| Video Output |  | 1Vp-p Composite Output (75Ω/BNC) |
| S/N Ratio |  | More than 50dB |
| Day & Night (Electronic) |  | Auto / Color / B&W |
| Min. Illumination |  | 0.1Lux @ F1.6 |
| Lens |  | 22X Optical Zoom |
|  |  | F1.6, f=3.9 ~ 85.8mm |
|  |  | Min. Working Distance:10-1000mm(Wide-Tele) |
| Menu | Camera ID | On / Off (001-254, fixed position) |
|  | Language | English, Chinese |
|  | Iris | Auto / Manual |
|  | Electronic Shutter | 1/50(1/60)s to 1/100,000s |
|  | Auto Gain | Low / High / Off |
|  | Auto Focus | Auto / Manual / Once |
|  | Day & Night (Electronic) | Auto / Color / B&W |
|  | Auto White Balance | ATW1 / ATW2 /Auto / Manual( Pre-set 3200K, Pre-set 5600K, R / G Gain Correction) |
|  | Motion Detection | On / Off (Zone, sensitivity) |
|  | Special Function | BLC, POS/NEG, Zoom speed(4 levels), Definition |
|  | Communication Interface | HIKVISION protocol, compatible of SONY VISCA protocol |
| Power Supply |  | 9-12V DC |
| Working Temperature |  | -10℃ ~ 60℃ |
| Power Consumption |  | 4W Max |
| Dimension (mm) |  | 50 × 55.6 × 86 |
| Weight |  | 235g |

## Table 3 DS-2CZ252P/N

| Parameter Model |  | DS-2CZ252P/N |
| --- | --- | --- |
| Image Sensor |  | 1/4 inch SONY Interline Transfer Super HAD CCD |
| Signal System |  | PAL/NTSC |
| Effective Pixels |  | PAL: 752 (H) × 582 (V) NTSC: 768 (H) × 494 (V) |
| Synchronization |  | Internal Synchronization |
| Horizontal Resolution |  | Color: 480 TVL |
|  |  | B / W: 520 TVL |
| Video Output |  | 1Vp-p Composite Output (75Ω/BNC) |
| S/N Ratio |  | More than 50dB |
| Day & Night |  | IR Cut Filter Auto Switch |
| Min. Illumination |  | Color: 0.2Lux @ F1.6 |
|  |  | B / W: 0.02Lux @ F1.6 |
| Lens |  | 23X Optical Zoom |
|  |  | F1.6, f=3.84 ~ 88.4mm |
|  |  | Min. Working Distance:10-1200mm(Wide-Tele) |
| Menu | Camera ID | On / Off (001-254, fixed position) |
|  | Language | English, Chinese |
|  | Iris | Auto / Manual |
|  | Electronic Shutter | 1/50 (1/60s) to 1/100,000s |
|  | Auto Gain | Low / High / Off |
|  | Auto Focus | Auto / Manual / Once |
|  | Auto White Balance | ATW1 / ATW2 /Auto / Manual( Pre-set 3200K, Pre-set 5600K, R / G Gain Correction) |
|  | Motion Detection | On / Off (Zone, Sensitivity) |
|  | Special Function | BLC, POS/NEG, Zoom Speed (4 levels), Definition |
|  | Communication Interface | RS-485, PELCO-D, Baud Rate, Address, PTZ Control (Optional Mode) |
| Power Supply |  | 12VDC, ±10% |
| Working Temperature |  | -10℃ ~ 60℃ |
| Power Consumption |  | 4.5W |
| Dimension (mm) |  | 68.7 × 66.1 × 125 (2.7” × 2.6” × 4.9”) |
| Weight |  | 550g (1.21lbs) |

## Table 4 DS-2CM252P/N

| Parameter Model |  | DS-2CM252P/N |
| --- | --- | --- |
| Image Sensor |  | 1/4 inch SONY Interline Transfer Super HAD CCD |
| Signal System |  | PAL/NTSC |
| Effective Pixels |  | PAL: 752 (H) × 582 (V) NTSC: 768 (H) × 494 (V) |
| Synchronization |  | Internal Synchronization |
| Horizontal Resolution |  | Color: 480 TVL |
|  |  | B / W: 520 TVL |
| Video Output |  | 1Vp-p Composite Output (75Ω/BNC) |
| S/N Ratio |  | More than 50dB |
| Day & Night |  | IR Cut Filter Auto Switch |
| Min. Illumination |  | Color: 0.2Lux @ F1.6 |
|  |  | B / W: 0.02Lux @ F1.6 |
| Lens |  | 23X Optical Zoom |
|  |  | F1.6, f=3.84 ~ 88.4mm |
|  |  | Min. Working Distance:10-1200mm(Wide-Tele) |
| Menu | Camera ID | On / Off (001-254, fixed position) |
|  | Language | English, Chinese |
|  | Iris | Auto / Manual |
|  | Electronic Shutter | 1/50(1/60)s to 1/100,000s |
|  | Auto Gain | Low / High / Off |
|  | Auto Focus | Auto / Manual / Once |
|  | Auto White Balance | ATW1 / ATW2 /Auto / Manual( Pre-set 3200K, Pre-set 5600K, R / G Gain Correction) |
|  | Motion Detection | On / Off (Zone, Sensitivity) |
|  | Special Function | BLC, POS/NEG, Zoom Speed (4 levels), Definition |
|  | Communication Interface | HIKVISION protocol, compatible of SONY VISCA protocol |
| Power Supply |  | 12VDC, ±10% |
| Working Temperature |  | -10℃ ~ 60℃ |
| Power Consumption |  | 4W MAX |
| Dimension (mm) |  | 61.2 × 50 × 88.3 |
| Weight |  | 370g |

## Table 5 DS-2CZ282P

| Parameter Model |  | DS-2CZ282P |
| --- | --- | --- |
| Image Sensor |  | 1/4"SONY Interline Transfer Super HAD CCD |
| Signal System |  | PAL |
| Effective Pixels |  | 752(H)×582(V) |
| Synchronous |  | Internal Synchronization |
| Horizontal Resolution |  | Day:540TVL |
|  |  | Night:600TVL |
| Privacy |  | 16 areas (area/position adjustable) |
| Video Output |  | 1.0Vp-p Composite Output(75Ω/BNC) |
| S/N Ratio |  | More than 52dB |
| Day & Night |  | IR Cut Filter Auto Switch |
| Min. Illumination |  | Day: 0.2Lux @ F1.4, 0.008Lux @ F1.4 (sensitivity×256) |
|  |  | Night: 0.02Lux @ F1.4, 0.0008Lux @ F1.4 (sensitivity×256) |
| Lens |  | Total Zoom 480X,Optical Zoom:30X, Digital Zoom:16X |
|  |  | F1.4, f=3.5 - 105mm Min. Working Distance:10-1500mm(Wide-Tele) |
| Menu | Camera ID | On/Off (OSD, 001-254, on the top right corner) |
|  | Language | Chinese /English |
|  | Iris | Auto/Manual |
|  | Electronic Shutter | 1/50S-1/10,000S |
|  | Auto Gain | Low/High/Close |
|  | Auto Focus | Auto/Manual/One focus |
|  | Day/Night | Auto/Day/Night |
|  | Auto White Balance | ATW1/ATW2/AC/Manual(3200K,6300K,R/G gain adjust) |
|  | Motion Detection | OFF/ON（area, sensitivity） |
|  | Special Function | Optical Zoom, BLC,Y-Value, C-Value, POS/NEG, ZSpeed (4 Class), Definition, Mirror, Digital Noise Reduction |
|  | Communication Interface | RS-485,PELCO-P/D, Baud, Address, PTZ Control |
| Power Supply |  | DC12V±10% |
| Working Temperature |  | -10℃--60℃ |
| Power Consumption |  | 4.5W MAX |
| Dimension (mm) |  | 124×66×68.5 |
| Weight |  | 450g |

## Table 6 DS-2CZ292P

| Parameter Model |  | DS-2CZ292P |
| --- | --- | --- |
| Image Sensor |  | 1/4"SONY Interline Transfer Super HAD CCD |
| Signal System |  | PAL |
| Effective Pixels |  | 752(H)×582(V) |
| Synchronous |  | Internal Synchronization |
| Horizontal Resolution |  | Day:540TVL |
|  |  | Night:600TVL |
| Privacy |  | 16 areas(area/position adjustable) |
| Video Output |  | 1.0Vp-p Composite Output(75Ω/BNC) |
| S/N Ratio |  | More than 52dB |
| Day & Night |  | IR Cut Filter Auto Switch |
| Min. Illumination |  | Day:0.2Lux @ F1.6,0.008Lux @ F1.6(sensitivity×256) |
|  |  | Night :0.02Lux @ F1.6,0.0008Lux @ F1.6 (sensitivity×256) |
| Lens |  | Total Zoom:576X,Optical Zoom:36X, Digital Zoom:16X |
|  |  | F1.6, f=3.4 - 122.4mm Min. Working Distance: 10-1500mm(Wide-Tele) |
| Menu | Camera ID | On/Off (OSD, 001-254, on the top right corner) |
|  | Language | Chinese /English |
|  | Iris | Auto/Manual |
|  | Electronic Shutter | 1/50S-1/10,000S |
|  | Auto Gain | Low/High/Close |
|  | Auto Focus | Auto/Manual/One focus |
|  | Day/Night | Auto/Day/Night |
|  | Auto White Balance | ATW1/ATW2/AC/Manual(3200K,6300K,R/G gain adjust) |
|  | Motion Detection | OFF/ON(area, sensitivity) |
|  | Special Function | Optical Zoom, BLC,Y-Value, C-Value, POS/NEG, ZSpeed(4 Class), Definition, Mirror, Digital Noise Reduction |
|  | Communication Interface | RS-485,PELCO-D, Baud, Address, PTZ Control |
| Power Supply |  | DC12V±10% |
| Working Temperature |  | -10℃--60℃ |
| Power Consumption |  | 4.5W MAX |
| Dimension (mm) |  | 124×66×68.5 |
| Weight |  | 450g |